Raspberry Pi Camera Module Setup Guide

Introduction

This guide walks through the complete setup process for a Raspberry Pi with an IMX290/IMX462 camera module, including device tree overlay configuration, verification steps, and debugging techniques.

Prerequisites

- Raspberry Pi 4 or 5 with Raspberry Pi OS (Bookworm or later)

- IMX290 or IMX462 camera module with CSI-2 ribbon cable

- SSH access to the Raspberry Pi or direct terminal access

- Knowledge of basic command-line operations

Part 1: Initial Hardware Setup

Step 1: Connect the Camera Module

- Power off the Raspberry Pi completely

- Locate the Camera CSI-2 connector (between the Ethernet and USB-C ports on Pi 4, or near the IO headers on Pi 5)

- Gently pull up the black retention clip on the CSI connector

- Insert the ribbon cable with the blue side facing the Ethernet port (or toward the back of the board)

- Push the retention clip down until it clicks

- Power on the Raspberry Pi

Step 2: Enable I2C Interface

The camera communicates via I2C, which must be enabled:

sudo raspi-config

Navigate to: Interface Options → I2C → Yes → Finish

Or edit directly:

sudo nano /boot/firmware/config.txt

Ensure these lines are present (uncommented):

dtparam=i2c_arm=on
dtparam=i2c_vc=on

Save: Ctrl+X → Y → Enter

Part 2: Configure Device Tree Overlay

The device tree overlay tells the kernel how to communicate with your specific camera model.

Step 3: Edit the config.txt File

sudo nano /boot/firmware/config.txt

For IMX290 Camera

Locate the camera auto-detection line and modify:

Find:
camera_auto_detect=1

Change to:
camera_auto_detect=0

Then add to the [all] section:
dtoverlay=imx290,clock-frequency=74250000,cam1
gpu_mem=128

Parameters explained:

- imx290 - Camera sensor type (use imx462 if you have that model)

- clock-frequency=74250000 - Clock frequency in Hz (74.25 MHz standard)

- cam1 - Camera port (use cam0 for single camera or alternative port)

- gpu_mem=128 - GPU memory allocation (128 MB minimum for camera)

For IMX462 Camera

If using the IMX462 variant, the overlay is similar but the sensor name differs:

dtoverlay=imx462,clock-frequency=74250000,cam1
gpu_mem=128

Step 4: Save and Reboot

After editing config.txt:

sudo reboot

Wait for the Raspberry Pi to restart (approximately 1-2 minutes).

Part 3: Verify Camera Installation

Step 5: Check Available I2C Buses

First, verify the I2C interfaces are loaded:

ls /dev/i2c*

Expected output (may vary):
/dev/i2c-1 /dev/i2c-20 /dev/i2c-21

Different buses represent different physical I2C connections. The actual camera I2C is internal (not directly numbered in /dev).

Step 6: Scan I2C Buses (Optional - See Debugging Section)

You can attempt to scan for I2C devices:

i2cdetect -y 20
i2cdetect -y 21

Note: On modern libcamera systems (Raspberry Pi OS Bookworm+), these buses may show unusual patterns because the GPU manages camera I2C directly. See Debugging section for explanation.

Step 7: List Available Cameras (MOST RELIABLE METHOD)

Use the libcamera utility to detect connected cameras:

rpicam-hello --list-cameras

Expected output for IMX290/IMX462:
Available cameras

0 : imx462 [1920x1080 12-bit RGGB] (/base/soc/i2c0mux/i2c@1/imx290@1a)
Modes: 'SRGGB10_CSI2P' : 1280x720 [60.00 fps - (320, 180)/1280x720 crop]
1920x1080 [60.00 fps - (0, 0)/1920x1080 crop]
'SRGGB12_CSI2P' : 1280x720 [60.00 fps - (320, 180)/1280x720 crop]
1920x1080 [60.00 fps - (0, 0)/1920x1080 crop]

Success indicators:

- Camera model is listed (imx290 or imx462)

- I2C address shows as @1a (standard I2C address)

- Resolution and frame rates are available

- Multiple color modes (SRGGB10_CSI2P, SRGGB12_CSI2P) are supported

Step 8: Test Camera Functionality

Take a test image to verify the camera works:

rpicam-still -o test.jpg

Or preview for 5 seconds:

rpicam-hello --timeout 5000

Look for a preview window (if using desktop) or confirmation that the image was captured.

Part 4: Debugging Steps

Issue: Camera Not Detected

If rpicam-hello --list-cameras shows no cameras:

Check 1: Verify Hardware Connection

Look for any error messages in kernel logs

dmesg | grep -i camera
dmesg | grep -i imx

If nothing appears, the kernel may not be detecting the camera hardware.

Check 2: Verify Device Tree Overlay Loaded

dtoverlay -l

Expected output should include:
imx290

If not listed, the overlay didn't load. Check config.txt for syntax errors.

Check 3: Verify GPIO and Power

Check if camera power pin is enabled

cat /proc/device-tree/base/soc/gpio@7e200000/camera_0_power/status

Should show "okay" if powered correctly.

Check 4: Verify I2C Communication

List all I2C buses and their drivers:

ls -la /dev/i2c*
i2cdetect -l

The camera should be on one of the i2c buses (typically shown as i2c@1 internally).

Check 5: Review Full Kernel Logs

sudo journalctl -u libcamera-server -n 50

Or check for media framework messages:

dmesg | tail -20

Issue: i2cdetect Shows All Addresses Responding

This is normal on modern systems and not a failure condition.

Explanation

Buses like /dev/i2c-20 and /dev/i2c-21 may show all addresses responding when probed with i2cdetect. This occurs because:

- GPU controls camera I2C directly - The Broadcom VideoCore GPU manages camera I2C communication, not the ARM processor

- Probing can interfere - Direct I2C probing may cause the GPU to respond with all addresses

- Not a camera issue - The camera is on an internal I2C bus (i2c@1) managed by the GPU, not on numbered buses like i2c-20

Solution: Ignore i2cdetect output for camera verification. Use rpicam-hello --list-cameras instead, which queries libcamera (the correct interface).

Issue: Camera Detected but Won't Capture

Check 1: Verify GPU Memory Allocation

grep gpu_mem /boot/firmware/config.txt

Should show:
gpu_mem=128

Or higher (256 for higher resolutions). Increase if needed and reboot.

Check 2: Check Camera Permissions

groups $USER

If "video" group is not listed:

sudo usermod -aG video $USER

Then log out and back in for changes to take effect.

Check 3: Test with libcamera-hello

libcamera-hello --timeout 5000

This is an alternative capture method that confirms libcamera functionality.

Issue: "Cannot open camera device"

Check 1: Verify Correct Camera Overlay

Ensure you're using the correct overlay name in config.txt:

- For IMX290: dtoverlay=imx290

- For IMX462: dtoverlay=imx462

Verify in config.txt:

grep dtoverlay /boot/firmware/config.txt

Check 2: Check for Duplicate or Conflicting Overlays

grep -i "dtoverlay.*cam" /boot/firmware/config.txt

Should only show one camera overlay. Remove duplicates or conflicting lines.

Check 3: Verify CSI-2 Connector Port

If using multiple cameras or if camera is connected to alternate CSI port, specify correctly:

dtoverlay=imx290,clock-frequency=74250000,cam0

(Change cam1 to cam0 or cam1 based on physical port used)

Part 5: Advanced Configuration

Using Higher Resolution

For 4K capture (if sensor supports):

rpicam-still -o image.jpg --width 3840 --height 2160

Check supported resolutions:

rpicam-hello --list-cameras

Look for available modes in the output.

Adjusting Sensor Parameters

Exposure control

rpicam-still -o test.jpg --shutter 10000 --gain 2

White balance

rpicam-still -o test.jpg --awb daylight

Brightness/contrast

rpicam-still -o test.jpg --brightness 0.2 --contrast 1.5

Recording Video

Record 10 seconds of 1080p video at 30fps

rpicam-vid -o video.h264 --width 1920 --height 1080 --framerate 30 --timeout 10000

Troubleshooting Checklist

Use this checklist when the camera is not working:

- [ ] Hardware is securely connected (ribbon cable fully inserted)

- [ ] sudo raspi-config shows I2C enabled

- [ ] /boot/firmware/config.txt has correct dtoverlay line with camera model

- [ ] GPU memory is allocated (gpu_mem=128 or higher)

- [ ] System has been rebooted since config changes

- [ ] rpicam-hello --list-cameras shows camera detected

- [ ] Camera address shows as @1a

- [ ] Sensor is listed as imx290 or imx462

- [ ] Resolutions and frame rates are available

- [ ] User is in video group (groups $USER includes "video")

- [ ] rpicam-hello --timeout 5000 runs without errors

Summary of Key Commands

Enable I2C (one-time)

sudo raspi-config

Edit configuration

sudo nano /boot/firmware/config.txt

Reboot to apply changes

sudo reboot

List available cameras (primary verification)

rpicam-hello --list-cameras

Check loaded device tree overlays

dtoverlay -l

Scan I2C buses (supplementary)

i2cdetect -y 20
i2cdetect -y 21

View kernel logs for camera

dmesg | grep -i camera

Capture test image

rpicam-still -o test.jpg

Preview for 5 seconds

rpicam-hello --timeout 5000

Record video

rpicam-vid -o video.h264 --timeout 10000

References

[1] Raspberry Pi Foundation. (2024). Camera Module Configuration. https://www.raspberrypi.com/documentation/accessories/camera.html

[2] Libcamera Project. (2024). Raspberry Pi Camera Support. https://libcamera.org/

[3] Broadcom VideoCore GPU Documentation. Device Tree Bindings for IMX290 Sensor.

[4] Raspberry Pi OS Bookworm Release Notes. (2024). https://www.raspberrypi.com/news/bookworm-the-new-raspberry-pi-os/